BitGuard-BNN: An Ultra-Low-Cost Stateful Binarized Neural Security Processor for Real-Time IoT Botnet Defense

(Abstract) Resource-constrained Internet of Things (IoT) edge devices are increasingly vulnerable to large-scale botnet attacks such as Mirai and Gafgyt. However, conventional deep learning-based intrusion detection systems (IDS) impose excessive computational burdens and suffer from high false alarm rates due to stateless per-packet evaluations. In this paper, we propose BitGuard-BNN, an ultra-low-cost, stateful binarized neural network (BNN) security processor tailored for real-time edge IoT botnet detection and defense. BitGuard-BNN integrates: (1) a lightweight payload-free streaming metadata engine extracting a unified 24-dimensional feature schema, (2) a cost-aware gated BNN that replaces floating-point multiplications with 1-bit XNOR-popcount operations while penalizing feature acquisition costs, (3) a hierarchical cascade combining a Boolean fast-path with a Tiny-to-Main BNN for early benign exit, (4) a dual-criterion confidence-distance open-set detector for zero-day threats, and (5) a 4-bit 5-counter temporal security state machine providing monotonic Level 0–5 defense recommendations. Rigorous zero-leakage evaluations on benchmark datasets (N-BaIoT and BoT-IoT) demonstrate that BitGuard-BNN reduces model memory by 95.9% (2.8 KB) and inference operations by 96.4% via 88.3% early exits, while achieving 99.64% Macro-F1, 99.98% high-risk attack recall, and a 92.4% reduction in false alarms per device-hour.

1. INTRODUCTION

The proliferation of IoT devices has led to widespread botnet infections (e.g., Mirai, Gafgyt) orchestrating crippling DDoS attacks and malicious reconnaissance [1]. While deep learning-based intrusion detection systems (IDS) achieve high classification accuracy, their floating-point (FP32) arithmetic and substantial memory footprints prevent deployment on microcontrollers with sub-watt power and limited RAM [2].

Furthermore, existing approaches suffer from major limitations: (1) heavy preprocessing overhead from extracting over 100 statistical features, (2) frequent false alarms and missed low-rate stealth attacks caused by stateless single-packet inference, (3) lack of safe, closed-loop mitigation mechanisms, and (4) vulnerability to unseen zero-day attacks [3].

To overcome these fundamental challenges, we present BitGuard-BNN, a hardware-friendly, stateful binarized neural security processor enabling real-time on-device botnet detection and active defense on ultra-low-power IoT edge nodes.

2. PROPOSED METHOD AND RESULTS

2.1. Streaming Feature Extraction and Cost-Aware BNN

BitGuard-BNN extracts a bounded 24-dimensional feature schema directly from packet headers without payload inspection. The classifier binarizes weights and activations to ±1, replacing floating-point multiply-accumulate (MAC) units with 1-bit XNOR and popcount bitwise operations [4, 5]. Inputs are encoded using 2-bit thermometer encoding. We formulate a composite loss penalizing differentiable feature gates: L = L_Focal + λ_feat * Σ σ(α_i) * c_i + β_FN * L_FN, physically pruning unselected feature groups.

2.2. Risk-Aware Multi-Stage Cascade and Open-Set Detection

Leveraging the predominance of benign traffic (>85%), BitGuard-BNN implements a 3-tier cascade: (1) Boolean Fast-Path: statistic threshold check dismissing trivial benign packets with zero neural operations. (2) Tiny-BNN Gate: 8-feature compact BNN for rapid benign filtering. (3) Main-BNN: multi-class classifier activated only for escalated suspicious packets. Exit scores are penalized by temporal risk to prevent adversary evasion. Unseen zero-day attacks are isolated as unknown_like using a dual posterior confidence and Mahalanobis distance metric.

2.3. 4-bit Stateful Security Machine and Active Defense

To eliminate transient noise and track stealthy low-rate probes, BitGuard maintains 5 saturating 4-bit counters (Scan, Flood, Beacon, Unknown, Benign, range [0, 15], 20 bits/device) with temporal exponential decay. The accumulated risk score drives a monotonic Level 0–5 mitigation engine (Allow, Log, Monitor, Rate-Limit, Isolate, Quarantine).

2.4. Hardware Folding and Edge Deployment

Batch normalization parameters are folded into integer dot-product thresholds (θ) and polarity bits, enabling pure bit-shift and integer comparison inference on FPU-less microcontrollers.

Table 1. Performance, compute reduction, and memory footprint comparison.

| Model Architecture | Macro-F1 | High-Risk Recall | Memory | Compute Savings |
| --- | --- | --- | --- | --- |
| FP32 MLP | 99.72% | 99.95% | 68.4 KB | 0% (Baseline) |
| Vanilla BNN | 99.15% | 99.68% | 3.2 KB | 78.5% |
| BitGuard (Ours) | 99.64% | 99.98% | 2.8 KB | 96.4% |

2.5. Experimental Evaluation and Analysis

Evaluated under strict zero-leakage partitions on N-BaIoT and BoT-IoT, BitGuard-BNN achieves 99.64% Macro-F1 and 99.98% high-risk attack recall with only 2.8 KB packed weights (95.9% reduction vs. FP32). Hierarchical cascade exits 88.3% of benign traffic early, reducing total compute by 96.4%. The 4-bit state machine cuts false alarms by 92.4% while mitigating low-rate stealth scans within 1.8 seconds.

3. CONCLUSION

BitGuard-BNN provides an ultra-low-cost, stateful binarized neural security processor designed for resource-constrained IoT edge devices. By unifying streaming metadata extraction, cost-aware BNN gating, early-exit cascades, and temporal state tracking, BitGuard resolves the computational bottleneck and false alarm dilemma of edge IDS, presenting a deployable architecture for next-generation on-device IoT security.

References

[1] Antonakakis, M. et al. Understanding the Mirai Botnet. Proc. USENIX Security Symp. 2017, pp. 1093–1110. (2017)

[2] Meidan, Y. et al. N-BaIoT—Network-Based Detection of IoT Botnet Attacks Using Deep Autoencoders. IEEE Pervasive Comput. 17(3), pp. 12–22. (2018)

[3] Koroniotis, N. et al. Towards the Development of Realistic Botnet Dataset in the Internet of Things for Network Forensic Analytics: Bot-IoT Dataset. Future Gener. Comput. Syst. 100, pp. 779–796. (2019)

[4] Courbariaux, M. et al. Binarized Neural Networks: Training Deep Neural Networks with Weights and Activations Constrained to +1 or -1. arXiv:1602.02830. (2016)

[5] Rastegari, M. et al. XNOR-Net: ImageNet Classification Using Binary Convolutional Neural Networks. Proc. ECCV 2016, LNCS 9908, pp. 525–542. (2016)